| Title | 104. Steps for Wheel Installation |
| --- | --- |
| Objective | To ensure a wheel never leaves our facility loose or unsafe. |
| Responsibility | Management Staff and certified technicians |
| Steps | 1. Line up the wheel and set flush on the vehicle’s hub. 2. With the right sized socket apply the lower lug nut by hand until the wheel is flush and secure. 3. Hand start the remainder lug nuts several threads. This is to ensure not to cross thread a lug nut. 4. Using a torque limiting stick closest to the vehicles lug nut torque specs (without exceeding) and an Impact, apply the lug nuts in a star pattern until the wheel is seated properly and secured. 5. After tightening all four wheels on the vehicle, lower the lift. 6. Have another tire rotation certified technician perform a final torque to all lug nuts on each wheel using a torque wrench that has been adjusted to the correct torque specification. (per manufacturer recommendations) 7. After pulling the vehicle out of the bay feeling for any issues or concerns, verify all tires are still seated properly. (If there is a concern address accordingly and repeat the steps) |
| Revision Date | 09/29/2023 |