| Title | 64. Lock Out-Tag Out Procedure General Environmental Controls |
| --- | --- |
| Objective | To keep all personnel safe whenever a piece of equipment needs to be shut down for maintenance or repair. |
| Responsibility | Authorized person initiating maintenance or repair. |
| Steps | - The person initiating the maintenance or repair must inform all employees in the service center that a specific piece of equipment is being shut down for maintenance or repair and no one is to attempt to use the equipment until after the maintenance or repair is complete. - The person initiating the maintenance or repair will disconnect the power source from the equipment to be maintained or repaired and place a locking device on the power source if applicable. The person initiating the maintenance or repair must hold the only key to remove the locking device. - If the power source cannot be locked out, the power source must be tagged out with a tag securely attached to the power source in such a manner that it must be removed in order to reactivate the equipment. The tag must contain the following information: - DANGER - DO NOT START - Person’s name that is tagging out the equipment. - The date the tag was placed. - After completing the maintenance or repair, the lock or tag will be removed and the equipment will be tested for proper operation. - After verifying that the equipment is working properly, the person that initiated the maintenance or repair will inform all personnel that the piece of equipment is back in operation. |
| Revision Date | 9/23/2023 |